Chapter One

# Introduction to Embedded Systems

# LEARNING OBJECTIVES

In this chapter the reader will learn:

    - What an embedded system is

    - Why to embed a computer

    - What functions and attributes embedded systems need to provide

    - What constraints embedded systems have

# CONCEPTS

An embedded system is an application-specific computer system which is built into a larger system or device. Using a computer system rather than other control methods (such as non-programmable logic circuits, electro-mechanical controls, and hydraulic control) offers many benefits such as sophisticated control, precise timing, low unit cost, low de- velopment cost, high flexibility, small size, and low weight. These basic characteristics can be used to improve the overall system or device in various ways:

    - Improved performance

    - More functions and features

    - Reduced cost

    - Increased dependability

Because of these benefits, billions of microcontrollers are sold each year to create embed- ded systems for a wide range of products.

    - Economics and Microcontrollers

Microcontrollers are remarkably inexpensive yet offer tremendous performance. The mi- croprocessor for a personal computer may cost $100 or more, while microcontrollers typi- cally cost far less, starting at under $0.25. Why is this so?

1

Microcontrollers provide extremely inexpensive processing because they can leverage economies of scale. MCUs are programmable in software, so a chipmaker can design a single type of MCU which will satisfy the needs of many customers (when combined with their application-specific software). This reduces the per-chip cost by amortizing the de- sign costs over many millions of units.

The cost of an integrated circuit (such as a microcontroller or a microprocessor) depends on two factors: non-recurring engineering (NRE) cost and recurring cost. The NRE cost includes paying engineers to design the integrated circuit (IC) and verify through simulation and prototyping that it will work properly. The recurring cost is in- curred by making each additional IC, and includes raw materials, processing, testing, and packaging.

The IC’s area is the major factor determining this recurring cost. The smaller the IC, the more will fit onto a silicon wafer and the lower the recurring cost. Microcontrollers are much smaller than microprocessors for personal computers, so they will cost less (given the same number of ICs sold). The NRE cost must be divided across each IC sold. As the number of ICs sold rises, the NRE adder falls, and so each IC’s price falls as well. Low- volume chips are more expensive than high-volume chips.

    - Embedded Networks

Some embedded systems consist of multiple embedded computers communicating across an embedded network, and offer further benefits. Each computer on the network uses a communication protocol to share the same set of wires to communicate, rather than dedicating one set for each possible communication route. Several advantages come from having fewer wires:

      - Lower parts cost, as fewer wires are needed

      - Lower labor costs, as it is faster to assemble

      - Greater reliability, as it has fewer connections to fail

Other advantages come from allowing separate nodes to share information. New features may be possible, or the system efficiency may be improved through better coordination of activities among different nodes.

# TYPICAL BENEFITS OF EMBEDDED SYSTEMS

As an example, let’s examine how embedded systems have affected automobiles. A typical modern car has dozens of microcontrollers embedded within it. Let’s see why.

    - Greater Performance and Efficiency

Computer control of automobile engines lowers pollution and increases fuel efficiency, re- ducing operating costs.

Burning gasoline with air in spark ignition engines is a tricky business if we want to max- imize efficiency, yet minimize pollution. The main factor affecting emissions is the ratio of air mass to fuel mass. The ideal ratio is 14.7 to 1, and the catalytic converter is designed to op- erate most efficiently at this ratio. If there is too little air (a rich mix), then excessive carbon monoxide (CO) and hydrocarbons (HC) will be produced. If there is too much air (a lean mix), then large amounts of oxides of nitrogen (called NOx) will be created. So we would like for each fuel injector to add just the right amount of fuel. This depends on the mass of the air inside the cylinder, which depends on factors such as air temperature and air pressure. These in turn depend on altitude and weather, as well as whether the engine is warmed up or not.

Another factor is the timing of the sparkplug firing. If it fires early, then there is more time for combustion within the cylinder before the exhaust valve opens. This raises the av- erage temperature within the cylinder and changes the combustion process, affecting CO, HC, and NOx concentrations.

It would be quite impractical to design a mechanical control system to consider all of these factors and squirt the fuel injectors at just the right instant for the right amount of time. Thankfully, an inexpensive microcontroller is quite effective at these kinds of calcu- lations and control.

    - Lower Costs

There are various ways in which an embedded system can reduce the costs associated with a device.

      - Component costs: Embedded software can compensate for poor signal quality, al- lowing the use of less-expensive components. For example, a low-cost pressure sensor may be very temperature-dependent. If ambient temperature information is already available, then it is a simple matter to compensate for the temperature- induced error.

      - Manufacturing costs: Many vehicles use the Control Area Network (CAN) pro- tocol to communicate across an in-car network. The embedded network reduces assembly and parts costs because of the simpler wiring harness.

      - Operating costs: As mentioned above, an embedded system enables automobile engines to operate more efficiently, reducing the amount of gasoline needed and hence lowering operating costs.

      - Maintenance costs: Some vehicles predict oil life by monitoring engine use his- tory, notifying the driver when an oil change is needed.

    - More Features

An MCU running application-specific software offers tremendous opportunities for fea- tures and customization. These features can make your company’s products stand out from the competition.

      - Cruise control keeps the car running at the same speed regardless of hills, wind, and other external factors.

      - Smart airbags reduce injuries by adjusting inflation speed based on passenger weight.

      - Power seats move to each driver’s preferred position automatically, based on whose keyless entry fob was used to open the car.

      - Headlights and interior lights shut off automatically after a time delay if the car is not running and prevents the lights from draining the battery.

    - Better Dependability

Embedded systems and networks offer many opportunities to improve dependability.

      - An engine controller (and other controllers) can provide various “limp-home modes” to keep the car running even if one or more sensors or other devices fail.

      - A warning of an impending failure can be provided.

      - Diagnostic information can be provided to the driver or service personnel, saving valuable trouble-shooting time.

# EMBEDDED SYSTEM FUNCTIONS

There are several common functions which embedded systems typically provide.

    - Control systems monitor a process and adjust an output variable to keep the process running at the desired set point. For example, a cruise control system may increase the throttle setting if the car’s speed is lower than the desired speed, and reduce it if the car is too fast.

    - There is often sequencing among multiple states. For example, a car engine goes through multiple states or control modes when started. During Crank and Start, the fuel/air mix is lean and depends on the engine coolant temperature. Once the engine has started, the controller switches to the Warm-Up mode, in order to raise the engine and exhaust system temperatures to their ideal levels. Here the fuel/air

mixture and ignition timing are adjusted, again based in part on the engine coolant temperature. When the engine has warmed up it can switch into Idle mode. In this mode the controller seeks to minimize the engine’s speed, yet still run smoothly and efficiently despite changes in loads due to air conditioning, power steering, and the electrical system.

    - Signal processing modifies input signals to eliminate noise, emphasize signal components of interest, and compensate for other factors. For example, a hands- free speakerphone interface may use multiple microphones, beam-forming, and active noise cancellation to filter out low-frequency road noise. Other sensors may have spark-plug noise filtered out.

    - Communications and networking enable different devices on the same network to exchange information. For example, the engine controller may need to send a message indicating speed. To do this, the speed value must be formatted according to the communication protocol and then loaded into the network interface periph- eral for transmission.

# ATTRIBUTES OF EMBEDDED SYSTEMS

Embedded systems are designed so that the resulting device behaves in certain desirable ways.

    - Embedded systems need to respond to events which occur in the environment, whether a user presses a button or a motor overheats. A system which is not suffi- ciently responsive is not likely to be a successful product. For example, when we press a channel select button for the radio, we would like for it to respond within some reasonable time.

    - For real-time systems, the timing of the responses is critical because late answers are wrong answers. Igniting the fuel in a cylinder is time-critical because bad tim- ing can damage or destroy engine components (to say nothing of reducing power, or the efficiency and pollution concerns mentioned previously).

    - Embedded systems typically require sophisticated fault handling and diagnos- tics to enable safe and reliable operation. Often the fault handling code is larger and more complex than the normal operation code. It is easy to design for the “everything goes right and works fine” case. It is far more difficult to determine methods to handle the exceptional cases. What is likely to fail? Which failures can lead to dangerous conditions? How should the system handle failures? How will you test that the system handles the failures correctly?

    - Embedded systems may be expected to operate independently for years with- out operator attention such as adjustment or resetting. The system is expected to

operate robustly and always work. Given that it is very difficult and expensive to write perfect, bug-free software, developers build in mechanisms to detect faulty behavior and respond, perhaps by restarting the system.

# CONSTRAINTS ON EMBEDDED SYSTEMS

Embedded systems often have constraints which limit the designer’s options, and can lead to creative and elegant solutions. These constraints are typically different from those for general-purpose computers.

    - Cost is a common constraint. Many applications which use embedded systems are sold in very competitive markets, in which price is a major factor. Often a manu- facturer will hire subcontractors to design and build individual sub-systems. This allows the manufacturer to pit potential subcontractors against each other, keeping prices down.

    - There may be size and weight limits for portable and mobile applications. An em- bedded system for an automotive remote keyless entry transmitter must fit into a small fob on a key ring which fits easily into a pocket. Similarly, the receiver must not be too heavy. A heavier car will have worse acceleration, braking, cornering, and fuel efficiency. Aircraft and spacecraft are especially sensitive to weight since a heavier craft requires more fuel to achieve the same range.

    - There may be limited power or energy available. For example, a battery has a lim- ited amount of energy, while a solar cell generates a limited amount of power. High temperatures may limit the amount of cooling available, which will limit the power which can be used.

    - The environment may be harsh. Automotive electronics under the hood of a car need to operate across a wide range of temperatures (-40°C to 125°C, or -40°F to 193°F), while withstanding vibrations, physical impact, and corroding salt spray. Spark plugs generate broadband radio frequency energy which can interfere with electronics.

# DESIGNING AND MANUFACTURING EMBEDDED SYSTEMS

Embedded systems are designed with a central microcontroller and other supporting elec- tronic components mounted on a printed circuit board (PCB). PCBs provide the means to connect these integrated circuits to each other to make an operational system. The PCB pro- vides structural support and the wiring of the circuit. The individual wires on the PCB are called traces and are made from a flat copper foil. While many circuit designs may use the same standard components, the PCB is often a custom component for a given design.

Designing a PCB requires a completed schematic for the circuit. This schematic is sometimes different than the schematic seen in textbooks. It often contains extra informa-

Figure 1.1 MSP-EXP430FR2355.

tion about the components and construction of the board. For example, a textbook schematic may only show a battery for the power supply, while the production schematic would show the battery case, the number of cells, the rating of the battery, and the manu- facturers of the components.

From the schematic, a designer will use their computer-aided design tools to identify the size of the PCB, and then place the electronic components on their board drawing. The designer will also place the wiring between the components on the PCB.

The physical PCB is manufactured from a fiberglass-resin material that is lightweight and inexpensive. A manufacturer will add copper wiring traces to the surfaces of the board and the appropriate insulation and white silk screening. The final manufacturing steps are applying solder, placing the components, and heating/cooling the solder. Figure 1.1 shows an example of a finished board.

The completed boards are electrically tested for functionality. Any board failing the test is inspected in more detail to find the problems. These boards are repaired to increase the passing rate acceptable to the manufacturer. The boards are wrapped in anti-static ma- terials and shipped to the next location for the next stage of production, or immediately in- stalled in a mechanical enclosure (like the mechanical shell of a mobile phone).

# AN EXAMPLE OF AN EMBEDDED SYSTEM: THE TEXAS INSTRUMENTS MSP430FR2355

The MSP430FR2355 is an example of a microcontroller from Texas Instruments based on their MSP430 architecture. A microcontroller’s architecture is the hardware design of the processor which determines what kind of instructions it is capable of executing and what hardware features are available. The MSP430FR2355 has <Insert stats here>

# SUMMARY OF BOOK CONTENTS

This book is structured as follows:

    - Chapters 2 and 3 present basic microcontroller concepts and the specifics of the RX62N.

    - Chapters 4 and 5 show how software is built. At the low level they show how pro- grams are compiled and downloaded. At the high level they show software engi- neering concepts and practices.

    - Chapters 6 through 8 present how to use peripherals to interface with the environ- ment and simplify programs.

    - Chapter 9 presents how to use interrupts to improve response times and program structure.

    - Chapter 10 presents how to use the floating-point math support in the RX62N.

    - Chapter 11 presents how to increase system robustness using the watchdog timer and voltage brownout detector.

    - Chapter 12 presents how to structure multi-threaded software so the resulting sys- tem is predictably responsive.

# RECAP

An embedded system is an application-specific computer system which is built into a larger system or device. Using a computer system enables improved performance, more functions and features, lower cost, and greater dependability. With embedded computer systems, manufacturers can add sophisticated control and monitoring to devices and other systems, while leveraging the low-cost microcontrollers running custom software.

# REFERENCES

Envisionnovation. EININC Products. Ontario: Envisionnovation, 2010. Web. Accessed at http://www.eininc. net/products.html

Numonyx MP25PE20 datasheet http://www.micron.com//get-document/?documentId=5965&file=M25PE20_ 0.pdf

RENESAS MCU: M16C Family/M16C/60 SERIES. Renesas Electronics America, Inc., 2010. Web. Accessed at http://am.renesas.com/products/mpumcu/m16c/m16c60/m16c6b/m16c6b_root.jsp

SKYWORKS Solutions, Inc. SKYWORKS Products: SKY65352–11. Woburn, MA: SKYWORKS Solutions, Inc., 2009. Web. Accessed at http://www.skyworksinc.com/Product.aspx?ProductID=752

https://www.ti.com/tool/MSP-EXP430FR2355?utm_source=google&utm_medium=cpc&utm_campaign=epd-null-null-GPN_EN_EVM-cpc-evm-google-wwe&utm_content=Tool&ds_k=MSP-EXP430FR2355&DCM=yes&gclid=Cj0KCQjw37iTBhCWARIsACBt1IwIpdThJ6-LdqkB3swg-D4cjWpAZV_U27xlUmC8480ckac73Tg-fAgaAr1sEALw_wcB&gclsrc=aw.ds